- Overall design / dimensions
The picture in the project description is conceptual, not a strict design specification. Please feel free to improve it. The platform should be large enough for an adult to stand comfortably and safely on it with a natural speaking posture. As a starting point, something around 60–70 cm in diameter or equivalent footprint seems reasonable. Compactness and portability are desirable, but safety and stability come first.

- Safety requirements / STO
Yes. Safety is a central design requirement. The active version should include at least an emergency stop, mechanical movement limits, fail-safe behaviour in case of power/control failure, and preferably a way to prevent unexpected startup. If the chosen drive architecture supports STO or an equivalent safe-torque-off concept, that would be very welcome. The prototype must never be able to generate sudden or violent movements.

- Maximum pitch angle
For the prototype, we do not want a mechanical bull. 😄 The challenge should come from subtle instability. A target of approximately ±3–5° tilt seems sufficient. Start smaller and test what already creates a meaningful balance challenge.

- Who buys it?
Our primary market hypothesis is professional rather than private use: public-speaking coaches, rhetoric/leadership trainers, communication centres, universities, training academies, and perhaps corporate learning departments. Whether there is also a viable individual consumer market is exactly one of the questions the Business team should investigate.

- Battery or outlet powered?
For the first prototype, mains power is perfectly acceptable and probably preferable. Standard 230 V supply can feed a safe low-voltage power/control system. Battery operation could become interesting for a later mobile product version, but it should not complicate the first prototype unnecessarily.

- Speech analysis
Please do not build a complete speech-analysis system from scratch. AllGoodSpeakers already works with acoustic speech-analysis technologies, and we can later provide or connect suitable metrics. For the prototype, the important task is to design the platform so that speech data/metrics can be integrated. A simple interface or mock integration is sufficient initially.

- Minimum dashboard metrics
For the balance part, I would consider the following the minimum useful set:

- left/right and front/back weight distribution

- deviation from centre / centre-of-pressure estimate

- sway amplitude

- sway path or movement over time

- platform pitch/roll

- recovery time after a perturbation

- overall stability score

Nice-to-have metrics could include movement smoothness, symmetry, response speed, and difficulty level.

For later speech integration, useful metrics could include speaking rate, pause behaviour, loudness, pitch variability, and possibly voice-quality/projection measures.

- Single-session or user history?
Minimum requirement: live feedback plus a session summary.
A stored user history would be highly desirable because training only becomes really interesting when users can see progress across sessions. For the prototype, local storage is sufficient; a cloud/server architecture is not mandatory.

- Existing research/material
Yes. We can provide relevant publications, background literature on public speaking, posture/body movement, speech breathing, charisma, and related AllGoodSpeakers technologies. We can also provide guidance on which speech and communication variables are scientifically meaningful. You do not need to start from zero.

- Most important outcome
The primary outcome is postural stability and controlled body behaviour during speaking.